# RISA Bot proposed UWB readiness zones

This is the revised mission map using the latest annotated drawing and the tunnel photo. The tunnel walls extend outside a constant-width road. Blue shows the first pass and outer loop; orange shows the second pass and parking. All 18 checkpoint numbers are preserved.

# Existing anchors retained

| Anchor | ID | Position in the upright drawing |
| --- | --- | --- |
| A1 | 1786 | Upper left |
| A2 | 1782 | Upper right |
| A3 | 1783 | Lower right, beside the start |

These placements come from the supplied track_anchors.json. It says survey_confirmed: false; its metric coordinates and antenna heights are examples. The map retains the corresponding drawing positions. The teal dot-dash triangle shows anchor geometry, not an RF reception boundary. No anchors have been moved or added.

# Ten physical areas eighteen mission checkpoints

Purple dashed outlines are proposed 2D readiness areas. A purple cross marks each area’s geometric centre. Centres can lie on grass or beside a road: they are references for zone membership, never steering targets. Two visits to the same area are distinguished by mission state.

Nominal footprints below are full width × full height in the upright plan. They are initial suggestions, rounded to 0.1 m using an approximate drawing scale of 110 pixels/m. The photographed drawing is not a metric survey, so these dimensions must be fitted to measured site coordinates before use.

| Zone | Area | Checkpoints | Suggested footprint | What to prepare |
| --- | --- | --- | --- | --- |
| Z01 | Start / return | 01, 13 | 1.9 × 1.3 m | Initial lane acquisition; on return, prepare the right turn and the second pass. |
| Z02 | Lane change | 02, 14 | 1.5 × 1.4 m | Prepare lane-change perception on either approach. |
| Z03 | Roundabout | 03, 15 | 2.2 × 2.3 m | Prepare roundabout perception and select the required exit. |
| Z04 | Tunnel | 04 | 1.8 × 1.8 m | Prepare tunnel lane following before reaching the portal. |
| Z05 | Left / NW turn | 05, 06 | 1.3 × 2.5 m | After tunnel exit, follow the left straight and prepare the northwest right turn. |
| Z06 | Hill complex | 07, 08, 09 | 3.2 × 1.1 m | Prepare the full uphill, summit and downhill behaviour together. |
| Z07 | STOP approach | 10 | 1.2 × 1.2 m | Expect the stop sign after the descent. |
| Z08 | NE / right side | 11, 12 | 1.5 × 2.3 m | Prepare the northeast right turn, then the right-side return. |
| Z09 | Parallel park / exit | 16, 17 | 1.5 × 1.6 m | Prepare parallel parking and retain task state while leaving the bay. |
| Z10 | L parking | 18 | 1.4 × 1.5 m | Prepare final L / perpendicular parking. |

# How anchor placement changes the design

The tunnel, roundabout and much of the lower/left route lie outside the drawn A1–A2–A3 triangle. This does not establish a blind spot or prove a particular error, but it is a reason to measure performance there rather than assume uniform 10 cm XY accuracy. Anchor geometry and obstructed paths influence UWB positioning quality; published work demonstrates these effects, although that study uses TDoA rather than this kit’s ranging implementation. Zhao et al., 2022.

Accordingly, Z03 and Z04 are broad areas with early preparation and latched behaviour. Prepare tunnel mode during the first roundabout exit. Inside the tunnel, local lane sensing and odometry confirm progress; UWB can support readiness when valid. These zone choices are a design proposal, not a measured coverage analysis. Do not simply enlarge zones indefinitely to compensate for bad ranges.

The hill needs special treatment: the supplied estimator assumes a fixed tag height of 0.35 m. On a ramp, the antenna’s height above the floor changes, so that flat-floor assumption can distort XY. Z06 therefore covers the whole hill. Identify uphill / summit / downhill using IMU pitch, vision and odometry; review the height model before trusting hill UWB fixes.

# Suggested trigger behaviour

Use 10 cm as a working error estimate, not a guaranteed bound. Zone footprints are deliberately much wider than 10 cm. Measure actual XY error, latency and dropout near each task, including the tunnel and hill.

Enable only the expected next area. Overlapping zones must not select a task by nearest centre. Z01 is reused for start/return, Z02 for two lane changes, and Z03 for two roundabout exits.

Enter once, then latch. As an initial bench setting, require about 0.2 s of fresh, consistent position observations before arming. Give the exit boundary an additional 0.2 m outward margin to reduce repeated entry/exit toggles. These are tuning suggestions; they do not guarantee rejection of a 10 cm error distribution.

Keep sufficient lead distance. Along the incoming route, allow at least speed × (preparation time + observation dwell + position age) plus measured along-track error margin before the task. Adjust the zone’s approach side for actual speed; the illustrated ellipses are starting shapes. Stop detection and braking have their own stopping-distance requirement.

Confirm actions using onboard sensing. A zone arms the behaviour; lane, sign, pitch and parking observations control execution and completion. Broad areas cannot resolve exact stop lines or parking poses.

Handle invalid fixes explicitly. With the current three-anchor package, a missing required anchor invalidates the UWB fix. Do not advance from stale coordinates or interpret disappearing UWB as an exit. Retain mission context and rely on valid local sensing; pause motion if that is also insufficient.

Reset repeated areas by mission progress. Do not re-arm the first-pass task when jitter re-enters a boundary. Second-pass permissions begin only after the outer loop completes.

# Checkpoint specific completion conditions

| Zone | State / confirmation rule |
| --- | --- |
| Z01 | Mission start enables CP01. Only completion of CP12 enables CP13; proximity to A3 is not a start/reset signal. |
| Z02 | Use first-pass / second-pass state. Camera/lane evidence determines the actual lane-change action. |
| Z03 | First visit: west/tunnel exit. Second visit: north/parking exit. Keep the selected exit latched until visually/odometrically confirmed. |
| Z04 | Pre-arm while leaving Z03 on the first visit. Keep behaviour active through the bend; a lost UWB fix is not a tunnel-exit event. |
| Z05 | Confirm tunnel exit and corner visually; preserve mission order when zone boundaries overlap. |
| Z06 | Use pitch, vision and odometry for the three phases. Pre-arm stop-sign detection before descent; do not wait for Z07. |
| Z07 | STOP position is provisional. Vision and speed/slope-aware braking determine stopping, not crossing the zone boundary. |
| Z08 | Arm only after the stop task completes; local sensing confirms the corner and straight. |
| Z09 | Enable only after the second roundabout visit. Parking success must be confirmed before CP17 can advance to final parking. |
| Z10 | Enable only after parallel parking succeeds and the bay is exited. Local sensing determines final pose and completion. |

# Geometry and files

The route and drawing outlines are reproducible vector geometry. The tunnel road uses concentric radii 60 and 120 in drawing pixels; wall strips extend to radii 48 and 132. Both the isometric view and plan view use the same corrected floor outline. The ramp alone raises the illustrated road; the tunnel does not.

trace_geometry.json stores drawing-pixel centres and outlines for reproduction, with explicit non-survey flags. It is not a ROS navigation map or a deployable geofence configuration. geometry_checks.json records route containment and coverage of all 18 checkpoints by their assigned areas. Exact STOP location and structure heights remain provisional.